|  |  | ### 13D SERIES |  |  |  |  |  |  |  |  |  | ## 1000VDC SINGLE OUTPUT ## 1 WATT DC-DC CONVERTER |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FEATURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  4 PIN SIP AND 8 PIN DIL PACKAGE.  1000VDC ISOLATION.  UNREGULATED OUTPUT TYPES.  UL 94V-0 PACKAGE MATERIAL. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | # GENERAL SPECIFICATIONS |  |  |  |  |  |  |  |  |  | ELECTRICAL SPECIFICATIONS |  |  |  |  |  |  |  |
|  |  OUTPUT RIPPPLE (20MHz Band limited) |  |  |  |  |  | : 150mVp-p MAX |  |  |  |  INPUT VOLTAGE RANGE |  |  |  |  |  | :±10% |  |
|  |  ISOLATION RESISTANCE |  |  |  |  |  | :1000MΩ MIN @500VDC |  |  |  |  OUTPUT VOLTAGE TOLERANCE |  |  |  |  |  | :±5% |  |
|  |  SWITHINGFREQUENCY |  |  |  |  |  | :100KHz TYP. |  |  |  |  LINE REGULATION(10% TO 100% F.L.) |  |  |  |  |  | :±1.2%/1% OF Vin |  |
|  |  OPERATING TEMPERATURE |  |  |  |  |  | :0 ℃ TO 70℃ |  |  |  |  LOAD REGULATION(10% TO 100% F.L.) |  |  |  |  |  |  |  |
|  |  ISOLATION VOLTAGE |  |  |  |  |  | :1000 VDC |  |  |  | 3.3V AND 5V OUTPUT TYPES |  |  |  |  |  | :15% MAX |  |
|  |  |  |  |  |  |  |  |  |  |  | 9V,12V AND 15V OUTPUT TYPES |  |  |  |  |  | :10% MAX |  |
|  | # 3.3v,5V,12V,24V,48V input types |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PART NUMBER |  | OUTPUT VOLTAGE (VDC) |  | OUTPUT CURRENT (mA) | EFFICIENCY (% TYP.) |  |  | PACKAGE STYLE |  | PART NUMBER |  | OUTPUT VOLTAGE (VDC) | OUTPUT CURRENT (mA) |  | EFFICIENCY (% TYP.) |  | PACKAGE STYLE |
|  | 13D-03S05N |  | 5 |  | 100 | 70 |  |  | 1 |  | 13D-03S05N3 |  | 5 | 100 |  | 70 |  | 3 |
|  | 13D-03S09N |  | 9 |  | 56 | 75 |  |  | 1 |  | 13D-03S09N3 |  | 9 | 56 |  | 75 |  | 3 |
|  | 13D-03S12N |  | 12 |  | 42 | 75 |  |  | 1 |  | 13D-03S12N3 |  | 12 | 42 |  | 75 |  | 3 |
|  | 13D-03S15N |  | 15 |  | 34 | 75 |  |  | 1 |  | 13D-03S15N3 |  | 15 | 34 |  | 75 |  | 3 |
|  | 13D-05S03N |  | 3.3 |  | 154 | 70 |  |  | 1 |  | 13D-05S03N3 |  | 3.3 | 154 |  | 75 |  | 3 |
|  | 13D-XXS05N |  | 5 |  | 200 | 70 |  |  | 1 |  | 13D-XXS05N3 |  | 5 | 200 |  | 70 |  | 3 |
|  | 13D-XXS09N |  | 9 |  | 110 | 75 |  |  | 1 |  | 13D-XXS09N3 |  | 9 | 110 |  | 75 |  | 3 |
|  | 13D-XXS12N |  | 12 |  | 84 | 75 |  |  | 1 |  | 13D-XXS12N3 |  | 12 | 84 |  | 75 |  | 3 |
|  | 13D-XXS15N |  | 15 |  | 67 | 75 |  |  | 1 |  | 13D-XXS15N3 |  | 15 | 67 |  | 75 |  | 3 |
|  | 13D-48S05N2 |  | 5 |  | 200 | 70 |  |  | 2 |  | 13D-48S05N4 |  | 5 | 200 |  | 70 |  | 4 |
|  | 13D-48S09N2 |  | 9 |  | 110 | 75 |  |  | 2 |  | 13D-48S09N4 |  | 9 | 110 |  | 75 |  | 4 |
|  | 13D-48S12N2 |  | 12 |  | 84 | 75 |  |  | 2 |  | 13D-48S12N4 |  | 12 | 84 |  | 75 |  | 4 |
|  | 13D-48S15N2 |  | 15 |  | 67 | 75 |  |  | 2 |  | 13D-48S15N4 |  | 15 | 67 |  | 75 |  | 4 |
|  |  |  |  | ### 13D SERIES |  |  |  |  |  | ## 1000VDC SINGLE OUTPUT ## 1 WATT DC-DC CONVERTER |  |  |  |  |  |  |  |  |
|  | ###### MARKINGS AND DIMENSIONS |  |  |  |  |  |  |  |  |  |  |  |  |  | ###### PIN CONNECTION |  |  |  |
|  | PIN; 4 PIN. 1; -INPUT. 2; +INPUT. 3; -OUTPUT. 4; +OUTPUT. |  |  |  |  |  |  |  |  |  |  |  |

| Dimensions:inches/mm Unless otherwise specified, all tolerances are±.005/±0.13 PIN; 8 PIN. 1; -INPUT. 4; +INPUT. 5; +OUTPUT. 7; -OUTPUT. |  |  |
| --- | --- | --- |